Step1: user needs to confirm if the extranet IP supplied by the supplier is available from the public network IP

① Input (ip.3322.org) on any client’s server computer browser address bar, enter key, then public network IP address shows……for example, 1.2.3.4

② Obtain the public network IP address on the router, for example: 1.2.3.4

Conclusion: if ①and② access to the same network IP address，the IP address is available public network IP address

③If it is not the public network IP address, request operator changing it to public network IP address.

Step2: user need to judge whether it’s a static public network IP address or a dynamic public network IP address

①User can apply for the domain name directly if it’s a static IP address

②If it is a dynamic IP address, user need to register a dynamic domain name and use dynamic DNS server to implement dynamic DNS.

③Dynamic DNS

If dynamic domain name registration is done, user can complete dynamic domain name resolution as below 2 ways:

1.Router side configure dynamic domain name service

2.Server side download software to configure dynamic DNS

User can choose a web site for dynamic domain name registration on below website

DYN Dynamic DNS Setup【www.dyndns.com】

3322 Dynamic DNS setupwww.pubyun.com】

Oray  DDNS setup 【www.oray.net】

We set DYN Dynamic DNS Setup as an example(figure on the picture)

Step3: register domain name

User can register domain name on Ali cloud site and choose paid or free domain name according to their needs. Registered domain name URL is www.aliyun.com

Step4: Domain name resolution

User need to resolve an address for the domain name which could be used when access customer TECARE server. User could resolve the domain name under the domain name management.

1.If the customer Internet address is a static public IP configuration, in domain name resolution, user need to add an A record and a MX record in domain name resolution.

2.If the customer Internet address is a dynamic public IP address., users are required to map the dynamic public IP address to the DDNS server in the router DDNS configuration or through the server installation of the DDNS software before configuring domain name resolution. We set DYN software as an example. The dynamic domain name I configured for DYN software is tecare.dyndns.net. Then the customer just needs to configure a MX record in domain name resolution.

Step 5: Port mapping

[port 7100] is the fixed end of the TECARE

[protocol TCP/IP UDP] is the protocol used by TECARE

[IP address] is the computer's static IP address of the TECARE server.

The client needs to log on to the router corresponding to the TECARE server and set the port map in the router

Step6: TECARE server configuration

①Domain name

Firstly, create the domain name in the TECARE server. Fill in the domain name in the ‘Name’ cell.

1.If user do not fill in the license number, user will be on probation for 30 days. User can create 10 accounts. As soon as the trial period arrives, only the first created accounts can receive calls but can't call. The remaining 9 accounts are out of use as shown:

2.If the license number is filled out ,the user is the official version as shown:

②Department

③Account

④Setting function

⑤Board management

Step 7: Fixed telephone - voice card configuration

①Users can configure the TECARE client directly If they do not use telephone as well as voice card and telephone exchange,

②If users use fixed telephone, they need to configure voice card and telephone switchboard.

③There are two kinds of voice cards that ECARE can use:

Intel 2P,4P,8P   dialogic diva analog media board PCI-E

FREE  2P

Insert the voice card into the PCI-E interface of the TECARE server PC board (PC board, PCI-E interface, and you can connect multiple voice cards in parallel

④TECARE voice card port type is FXO[not feeding]

RJ10 --FXO--2P

RJ11 --FXO--4P

RJ45 --FXO--8P

TECARE uses the voice card FXO (not feeding) interface, connect the PBX FXS (feed) interface, then the phone can establish the call.

Step8: Fixed telephone ---- telephone switchboard configuration

①The telephone switch (PBX) port has 2 types:

RJ11--FXO (not feeding)

RJ11--FXS (feeding)

②There are three modes of ringing in the telephone switchboard. TECARE uses the computer on duty mode and the manual on duty mode

Full sound mode--external access then each internal ring will ring

Computer on duty mode-- the caller needs to enter the extension number

Manual on duty mode—front desk transfer the call for you

③Ordinary telephone exchanges can be set up (relay number - blind line transfer). When one of our ports is busy, the telephone exchange can provide free ports for customers and establish connections.

Advanced telephone exchanges can be set up (trunk numbers - resilient packets). Telephone exchange can automatically connect us to an idle port and establish a connection.

Note: the other end of the RJ11--FXS (feed) interface can not be the FXS (feed) interface, otherwise it will burn all the FXS interfaces of the PBX.

Step9: TECARE client configuration

- Login

- Main interface

- Address book

- System personal settings

- Advanced settings

Step10: Customer log on successfully

Clients run the client, enter the TECARE account number and password, then log on to the TECARE server as shown

Step11: Successful call

①The customer picks up the fixed telephone, then enters the server extension number in turn. The customer binds the extension number, the hot key number, the telephone password (optional) establishes a conversation with the other party.

②Customers directly enter other party's hotkey number or TECARE account to establish a conversation as shown: