TECARE Enterprise Version

Product Description

Version 1.0

.

TEWARE, INC

Contents

1.	TECARE Summary	6

1.1	TECARE R&D Background	6

1.2 What is TECARE	7

1.3	TECARE Advantage	7

2.	TECARE Communication Protocol Flows	8

2.1	TECARE Network Architecture	8

2.2	TECARE Communication Process	8

3	TECARE Installation	9

3.1	TECARE Operating Environment	9

3.2	Sever Installation	10

3.3	Client Installation	10

4.	TECARE Product Instructions	12

4.1	Sever Instructions	12

4.1.1	Sever Main Interface Instructions	12

4.1.2	Sever Main Interface Instructions	12

4.2	Client Instructions	17

4.2.1	Client Main Interface Instructions	17

4.2.2	Client Function Detail Explanation	21

4.2.3	TECARE Calling Party Instruction	27

5.	Consulting and purchasing	30

## TECARE Summary

  - TECARE R&D Background

In the days of information and networks improvement, the raise of Web phone has became an inevitable trend .Web phone is also known as VOIP(Voice Over Internet Protocol) phone, Which is a global telephone call through the Internet. VOIP can be divide into Soft phone and Hardware phone. People install a software and make a call through headset on Internet called Software phone. Hardware phone use a voice gateway which connect Router and formal telephone to make a free call through the Internet. VOIP will ensure the quality of connection and reduce the cost greatly. The function of VOIP also can be extended unlimitedly. That makes it more popular and become the most main stream communication mode. It’s foreseeable that the future of Telecom Service will show a diversified pattern. Voice Service maybe will be provide by PSIN network (traditional telecom network), Internet or other network .Also ,users’ choice will include computer to computer, computer to phone ,phone to phone, phone to smart mobile and etc. This is the inevitable result of conforming the development of communication network.

So far, VOIP product could be divided into two basic modes: 1,Zero tariff with high requirements of Hardware(the cost of hardware generally not acceptable),and users should be head office and branch office.2,Low tariffs means constant recharge or earn free communication by participating in the software producers’ activities. With the development of technology, breakthrough product will springing up inevitably. TECARE is the first one which could make free call between software and software, software and phone, phone and phone with low cost in the world!

1.2 What is TECARE

TECARE® is a worldwide VOIP software that provides calling and answering two-way permanently zero tariffs with high quality calls which is developed by Dalian TEWARE INC. As a new product in VOIP filed, TECARE has re-upgraded the web phone software. It adopt Sever Decentralization mode (figure on 2.1) to make an access through computer and computer, computer and phone, phone and phone. TECARE is the first VOIP software which permanently realize zero tariff between phone and phone, phone and web phone, web phone and web phone (both side all use TECARE) with low cost. In future version, TECARE will make you zero tariff to talk freely among the world.

  - TECARE Advantage

TECARE has two major breakthroughs. Firstly, if two sides all use TECARE, they will make free call forever. It’s TECARE’s core value and unique in the world. Secondly, TECARE not only realize dial between web phone and web phone, web phone and phone, but also realize zero tariff between phone and phone. It also the first software in VOIP field.

TECARE advantages are mainly reflected in the following eight aspects:

- World’s first permanent zero tariff

Worldwide: You could use TECARE to make a free call with any countries and regions without territorial limitation.

Pioneer: TECARE is the first software that base on the traditional analogue signal phone to do Internet addressing besides web phone and digital signal phone.

Permanent zero tariff: TECARE realize calling and answering two-way permanent free call without time quantum limitation. It’s a true significance zero tariff communication software.

- Multi-terminal

TECARE support multi-terminal call between different kinds of web phone and phone such as. PC terminal, Mobil terminal (future version provide), digital signal phone, analog signal phone etc.

- Smooth transfer to existing Internet

On the basis of compatible PSTN communication mode, TECARE realize TE mode’s VOIP free call. So it will enter enterprise’s existing network smoothly without changing network environment. It’s the greatest assurance solution to the client who choose TECARE first time.

- Server decentralization ensure data security

Users have the initiative by dispersing server. Each TECARE user will have own server, which will storage all the data and charge by themselves. It improve date security greatly.

- Double significant Instant transmission

Instant transmission of TECARE is not only the immediately ring up when call comes, but also could be set ring up simultaneously both on the computer and phone. User could choose a convenient way to answer.

- Utilities and commonality

TECARE support one key one quick call. It’s suitable for all kinds of people without territorial limitations.

- Multichannel call

TECARE support five channel calls at most one time

- Low cost

TECARE running cost=cheap hardware cost + low price software

## TECARE Communication Protocol Flows

  - TECARE Network Architecture

TECARE currently running version is TECARE 1.0 (Enterprise Versions). It includes sever side and client side. Sever side is responsible for competing the registration, storage and manage user’s account message. When user login TECARE, sever side will authenticate to guarantee the global uniqueness of the TECARE No. . User will login after identify authentication. The network structure of TECARE is unique mode of operation. It’s different from telecom networks and other networks. TECARE servers are distributed. Each user has own server and each server connect network switch and telephone exchange (figure 2-1).

Figure 2-1 TECARE Network Architecture

  - TECARE Communication Process

Below is process of communication

- Phone call phone

- Web phone call phone (phone call web phone)

- Web phone call web phone

Tips: In practical situation, called party’s web phone and phone could be set ringing at same time in above 3 communication modes. User could choose any way to answer according to their personal situations.

## TECARE Installation

## TECARE Operating Environment

- Hardware Requirements

Computer, telephone switch, network switch, Router, phone (optional)

Sever: Intel/AMD central processing unit, CPU1.5Ghz, Memory 4GB, 100G free hard disk space, Network card, voice card (phone necessary, refer to official website for detail voice card information).

Client: Intel/AMD central processing unit, CPU1.5Ghz, Memory 1GB, 10G free hard disk space, Network card, , sound card, headset and microphone.

- System requirements

Sever: Microsoft Windows sever 2003/2008/2016/2016(above system only support 64 bits)

Client: Microsoft Windows XP/Vista/Win 7/Win 8/Win 10(support 32 bits and 64 bits )

- Port setting requirements

Sever port: 7100 in TCP communications

7100 in UDP communications

- Server requirements: Public IP addresses that can be accessed.

【Note】It’s good to access static IP address. If access dynamic IP address, you need to use the third party DDNS (Dynamic Domain Name Sever) or other tools to DDN resolution .

Explanation: DDNS is a dynamic IP address mapping of users to a fixed domain name service. Each user connect to the network when the client go through the transmission of information.

  - Sever Installation

To use TECARE software, users need to install TECARE server first. The steps are so follows:

- Login website: www.teware.net to download TECARE server installation package. After installation, double click TECARE server installation icon, choose install language (figure 3-1), then click “Next”.

Figure 3-1

- Run TECARE Server Setup Wizard and click “Next” (Figure 3-2)

Figure 3-2 Server Setup Wizard interface

- Please read agreement carefully (figure 3-3), click “Accept”, then click “Next”.

Figure 3-3 Server installation agreement interface

- Choose the installation path (full English directory required, the default path is Disk: C), 【Figure 3-4】then click “install”.

Figure 3-4 Patch selection interface

- When the progress reached 100%, click “Next”.( Figure 3-5)

Figure 3-5 Installation progress interface

- User should choose among “Initialization software”, “Start immediately”, “Start with windows” and “Add shortcut”. First two items couldn’t be chosen at same time. Then click “Finish” (figure 3-6) the server installed.

Figure 3-6 Server Installation complete interface

  - Client Installation

After installing TECARE server, user need to install the TECARE client as follows:

- Login website: www.teware.net to download TECARE client installation package. After installation, double click TECARE client installation icon .Start TECARE client installation wizard (figure3-7),click “Next”.

Figure3-7 Client Setup Wizard interface

- Please read agreement carefully (figure 3-8), click “Accept” if users feel okay, then click “Next”.

Figure3-8 Client installation agreement interface

- Click “Browse” to choose the installation patch (default path is Disk: C) 【Figure 3-9】. Then click “Install”.

Figure 3-9 Client installation path selection interface

- When the progress reached 100%, click “Next” (figure 3-10).

Figure 3-10 Client installation schedule interface

- User should choose among “Start with windows”, “Start immediately” and “Add shortcut”. Then click “Finish” (figure 3-11), the server installed.

Figure 3-11 Client Installation complete interface

- TECARE Product Instructions

  - Sever Instructions

    - Sever Main Interface Instructions

Double click TECARE server icon  and enter TECARE server main interface. Click top left corner “Option-Language” to choose system language. Set simplified Chinese as example (Figure 4-1).

Figure 4-1 Client main interface

    - Sever Main Interface Instructions

The main function of TECARE server is the management for enterprise domain name, department, account setting etc.

- Domain Name

Click “Domain Name” could add, delete, change, search domain name (figure 4-2). Click “Add” then the button change to “Cancel” and “OK”. Fill the domain name in the information bar behind the “Name” bar (request letter, number, minus, English period, 3 to 62 characters) and description (optional). Number of users under domain name will be generated automatically. Enter the authorized TECARE license No. (optional in trial time). Click “OK” as domain name add successfully (figure 4-3). If user wants to delete, change, search domain name, just select the domain name above the button and process.

【Note】:TECARE software trail period is 30 days. Maximum number of users is 10.

Figure 4-2 Domain Name Interface

Figure 4-3 Domain Name addition interface

- Department

In the expanded tree menu of “TECARE management system”, users could add, delete, change, search department by click “Department” button (figure 4-4). Click “Add”, the button change to “Cancel” and “OK”. Fill the department name description (optional) in the information bar, and select the department’s domain name in the drop-down list. Number of users refer to the users under domain name will be generated automatically. “Enable Department Logogram Call” refers to when users under department call others, answer party will get the logogram of calling department. Other users also could call by department logogram. After enable and fill in the department logogram (require case insensitive letter, number, under line, 1 to 16 characters), the department logogram TECARE ID is generated. “Enable Identify Transparency Mode” is the exception of department logogram. “Enable Identify Transparency Mode” refers to user under current department calling others, the answer party have same domain name with call party, answer party got the caller TECARE ID instead of department logogram. “Department Identify Transparency Mode” means if caller and answers belongs to same department, answer will got caller’s TECARE ID instead of department logogram. Click “OK” and department addition complete (figure 4-5, HR department’s TECARE logogram is hr@tecare.net). Users could select the department above the button to delete or change department.

Figure 4-4 Department interface

Figure 4-5 Department addition interface

- Account

In the expanded tree menu of “TECARE management system”, click “Account” to add, delete, change, search, account on client (figure 4-6). Click “Add” then button change to “Cancel” and “OK”. Fill the user’s information in information bar. It needs to be explain as follows. ①User name is the prefix of TECARE ID, letters (case-insensitive), numbers, under line, 1 to 16 characters. ② Name is user’s remark name.③ “Reset login password” is used for changing client user’s login password (used for log on client) to default password. Login password is “000000” for first login client. ④ Extension No. is the No. tie to TECARE is phone’s extension No. . Demand for numbers. ⑤ “Reset Extensional Phone Password” is used for changing client extensional phone password(the password when users call by telephone is the client password) to default password. The default password is “000000”. ⑥How to choose domain name? Select the domain name which will be added, click , add it to selected domain name(user may choose more than one option). How to delete domain name? Select domain name which going to be deletead, click. After setting up, click “OK” and the account has been added in TECARE. TECARE ID has been generated as “account @ domain name” (figure 4-7 user Xuming’s TECARE ID is xuming@tecare.net). Select the account name above the button and process if user want to change or delete account.

【Note 1】:Account of users in TECARE account depends on the authorize license No. The upper limit during probation is 10.

【Note 2】:User’s login password and extensional phone password just could be changed by user themselves on client side. Server administrator just could rest password to default.

【Note 3】:If user choose more than one domain names it means user has several TECARE IDs. User could choose anyone to login.

【Note 4】:User ID is unique in one server. That can’t be duplicated.

Figure 4-6 Account interface

Figure 4-7 Account addition interface

- Settings

In the expended tree menu of “TECARE Management System”, click “Settings”(figure 4-8) to enable department settings, log settings, DNS settings and phone settings. 1) Department settings: “Delete Department and Delete User” →delete department as well as the users under the department. 2) Record settings→ We recommend to enable record settings. Record settings also mean to keep communication records. ‘’Delete Expired Record” function refers to system will delete the communication record expire “Keep Record Days” automatically. “Keep Record Days” could be any number among 1 to 99999. 3) DNS Settings→Click “Acquisition” to get DNS server ID address. User could fill in the available by hand and set addressing timeout function. Number is required. 4) Phone settings: “Extension Phone Figures” means the phone number figures which tie to TECARE. Click “Apply” the setting is done.

Figure 4-8 Setting interface

Figure 4-9 Settings complete interface

- Management

In the expended tree menu of “TECARE Management System”, click “Management (figure 4-10)”. It includes card management, phone management and record management.

Figure 4-10 Management interface

- Card management

Click “Card Management 【figure 4-11(1)】, import effective card interface file(please check the card’s brand, model, effective file on official website). If server starts , user could see each port’s running condition 【figure 4-11(2)】.

【Note】:When user start card management, they should make sure current software system is not running.

Figure 4-11 (1) Management-Card Management interface

Figure 4-11 (2) Management-Card Management interface

- Phone Management

Click “Phone Management”, open “Phone State” interface by default (figure 4-12). Interface surfaces bath sides’ information on the line. It includes c-t, c-c, t-t, t-c models. Set c-t model as an example. It means calling party use computer and answering party use telephone. Administrator could select either of parties and cut of call by “Cut” button.

Figure 4-12 Management-Telephone Management-Phone state interface

Click “Black and white list management” to set other users calling authority (figure 4-13). If user set black list, other user in black list calling request will be restricted. Click “Add” to add user in list. ① Add user name: this user’s call will be restricted under all domain names. ② Add domain name: users calling will be restricted under this domain name. ③ Add TECARE ID: only this user calling will be restricted. Click “OK” to complete black list addition. If user set white list, only users in this list’s calling will be approve. Additional method is same with black list. If users want to edit or delete list, select list and process.

Figure 4-13 Management –Telephone Management-Black and white list Management interface

- Record Management

- Click “Record Management” to check or export specific date’s communication record. Operation method: Select date, click “Search”, the communication record will display. Click “Export” to export the l communication record during specific period. (figure 4-14).

【Note】:User should start “Record Management” in settings firstly then it will be available.

Figure 4-14 Management-log management interface

If server settings are okay, please click  button and server will start running.

  - Client Instructions

    - Client Main Interface Instructions

Double click TECARE server icon  and enter login interface (figure 4-16).

Figure 4-16 Login interface

TECARE provide 2 login options: general login and advanced login.

Option 1: General Login (figure 4-17): Please fill in the TECARE ID (ex: xuming@teware.net) and password. Choose ticking “remember password” and “Auto-login” or not. Then click “Login”.

Option 2: Advanced Login (figure 4-18): Tick “Advanced Login” and fill in the sever address (IP address or main engine name under one domain name), TECARE ID and password on new popup interface. Choose ticking “remember password” and “Auto-login” or not. Then click “Login”.

【Note】:One computer only could run one TECARE client at one time. One TECARE ID could login different computer but cannot log on several computers at the same time.

Figure 4-17 Login interface

Figure 4-18 Login interface

After login (figure 4-19), enter TECARE client main interface 【Figure 4-20(1) 】.

Figure 4-19 Logging interface

Figure 4-20(1) Login interface

The main interface is derided into two regions: display region and keypad region【Figure 4-20(2) 】.

Display region: Display current channel, model, communicate time, other party’s information and current phone state.

Keypad Region: There are different meaning of numbers around software main interface. 【Figure 4-20(2) 】.

- Four channel keys and one communication model key. 2) Dialing and answering key. 3) Hung up key. 4) Ring dial key. 5) Delete key. 6) Transfer key. 7) Cancel key. 8) Function setting key.

Function details as follows:

- Four channel keys and one communication model key.

Four channel keys are ①key, ②key, ③key, ④key on main interface

Functions: Channels load phone voice. Every channel load on channel speech. So TECARE will make 4 calls at one time. Every channel has 4 conditions respect different communication situations. It reflects by different colors of button right corner indicator and current phone states. In details:

| Condition Item | Indicator | Call status display | State interpretation |
| --- | --- | --- | --- |
| 1 | Transparent | No display | Channel is free |
| 2 | Green | Start Communication | Channel is busy |
| 3 | Yellow | Calling | New call coming |
|  |  | Call hold | Call hold, Answer anytime |
| 4 | Yellow twinkle | Wait | Away in communication. wait for answering |

Usage method: Click channel button and “Current Channel” will display channel No.. Then process will the following operation.

●Communication Mode key: Middle position display TE or PSTN key.

Function: switch communication models. The software includes TE and PSTN communication models. TE stands for TECARE model. It’s the major model for free call. PSTN is short for public switched telephone network. It’s the traditional telephone network. TECARE is compatible with current telephone models. Calling under this model will be billing by telecom operations. PSTN is totally same with traditional telephone model. No necessary to explain in text.

Usage method: Click “Switch” key to switch between two models.

- Dialing / Answering key

Function: Dial or answer calls.

Usage method: Figure 47-48 page.

- Hung up key

Function: Reject or stop calling.

Usage method: Select the channel going to cut and click “Hung up” key. If user click “Hung up” key directly means to stop current channel communication.

- Ring dial key: Calling party’s server connect with calling party’s phone actively. It’s simply the dialing process when make a call by telephone (For better understand “Ring dial”, we will make an explanation when dialing with TECARE: At most time, calling party will dial server voice interface card No. by phone first. After connecting with calling party server then keep dialing with answering part. “Ring dial” button replace the process of calling party telephone connect to calling party server to simplify dialing steps. Dialing method will be explained in detail in page 46).

Usage method: When calling party dial with telephone, click “ring dial” button first. After screen shows “Ring dial successfully”, calling party’s phone will ring up .Users could pick up the phone and dial to answering party.

- Delete key

Function: Clean up main interface contest one by one, same with backspace.

- Switch key

Function: Transfer calling.

Usage method: When connect with calling party, fill in the answering party’s TECARE No. or hotkey, then click “transfer” key, the call will be transferred successfully.

- Cancel key

Function:  Empty information in display interface.

- Function setting key

Five icons from left to right represents: Address list, Audio frequency, System settings, Call record, Soft keyboard. (See page 31-45 for details)

【Note 1】: The usage method in this instruction is just for TE Model. Calling under PSTN model could totally same with traditional telephone. No need to explain in instruction.

【Note 2】:Calling under TE model is free. Calling under PSTN model will be billing by telecom operators.

Figure 4-20(2) TECARE server main interface

    - Client Function Detail Explanation

- Address list

Click  button to open TECARE address list main interface (figure 4-21). Users could add, edit, delete, search address list.

Figure 4-21 Address list main interface

- Addition: Click “Add” to open addition interface. The button change to “OK” and “Cancel”. Please fill in the user’s information 【Figure 4-21(1) and Figure 4-21(2)】.User should entry at least one in the three names of family name, name and company without @ character. If user owns more than one TECARE ID, please click  button to add one by one. Hotkey is user-defined number which fill in by themselves according to their hobbits. Hotkey for different users in address list is unique and can’t be repeated. Click “OK” and user is added successfully.(figure 4-23)

【Note】: After being added in address list, when the call coming, the user’s information will be display in priority order as follows: user name, company and TECARE No.

Figure 4-22(1) Address list-user addition interface

Figure 4-22(2) Address list-user addition interface

Figure 4-23 Address list-user addition complete interface

- Edition: Select user need to be edited and click “Edit” to change information.

- Delete: Select user need to be deleted, and click “Delete”.

- Check: Select user need to be searched and Click “Check”.

【Note】: When the hotkey duplicate with extension No. , it will be dial as hot key in TE model and extension No. in PSTN model by default.

After setting up address list, click red indicator button to close it.

- Audio frequency

Click  button on client main interface (figure 4-24), slide the sliders of mike and headset to adjust the audio frequency.

Figure 4-24 Audio frequency adjustment interface

After setting up audio frequency, click red indicator button to close it.

- Click the  button on client main interface to enter system setting up interface.(Figure 4-25)

Figure 4-25 System setting interface

- Personal settings

Click “Personal settings” > “Account settings” (figure 4-26). If user have several TECARE No., he could select one of them available domain name and its responding TECARE No. as display number when make a call.

Figure 4-26 System setting-Personal settings-Account settings interface

Click “Personal settings > password setting” to change login password and enabled or modified phone password (figure 4-27). Login password refers to the password used by personal users when they log on the software client. The password requirements are 6-16 letters (case sensitive) or numbers. When the user logs on first time, the default login password is 000000. Phone password used for call by phone (refer page 46 phone dial method). Users can choose use it or not. Phone password requires 1to16 numbers. When the users enable it first time, the default password is 000000. When the changes are complete, click “OK” (figure 4-28).

【Note】: If forget login password or phone password, users could apply to administrator for reset the password on the serve.

Figure 4-27 System settings-Personal settings- Password settings interface

Figure 4-28 System settings-Personal settings- Password settings inter face

- Advanced settings

Click “Advanced settings> General settings (figure 4-29). Users could setup for starting, exiting ringing modes for TECARE. The ringing modes mean that when a call comes in, user could select software ringing or phone ringing even both all ringing.

Figure 4-29 Advanced settings-General settings

Click “Advanced setting > Black and white list settings”(figure4-30) to set incoming user permission. Enable black list means that call request in the black list will be restrict. Usage method: click “Enable black list” button>click “add” (figure 4-31) to add behind “TECARE No.” by 3methods. ①Fill in user name means this user’s call will be restrict under any domain name. ② Fill in domain name means all the user under this domain name will be restrict. ③ Fill in complete TECARE No. means only this TECARE user will be restrict. After filling in “Instructions” (optional), click “OK”, black list setup successfully. Enable white list means only the call request in white list is allowed. Usage method is same with black list. If users want to edit or delete the list, select the appropriate list, click the button to process.

.

Figure 4-30 Advanced setting- Black and white list interface

Figure 4-31 Black and white list Addition interface

After setting up system, click red indicator button to close it

- Call Records

Click the  button on client main interface to open call records interface【figure 4-32(1)】,  4 icons represent the missed call, received call, TE model dialed call, PESTN model dialed call. Click one of them【figure 4-32(2)】, the selected call records will display 3icoms  on the right. They mean call this user, add this user to address list, delete the record.

Figure 4-32(1) Call records interface

Figure 4-32(2) Call records expended interface

After setting up call record, click red indicator button to close it.

- Soft keyboard

Click the , icon on client main interface to use soft keyboard(Figure 4-33). Click the red indirect button to close it

Figure 4-33 Soft key board expended interface

    - TECARE Calling Party Instruction

In TE mode, when calling party make a call by different phones, TECARE dialing methods is divide in to below conditions (figure sheet 1)

| Calling party | Answering party | Dialing methods |
| --- | --- | --- |
| Web phone | Web phone or telephone | Mode 1: First click an idle channel key. Then fill in the answering party’s TECARE No. or TECARE hot key on main screen. Finally click the call/ answer button, the phone dial out. Mode 2: First select an idle channel . Then open address list and Double click answering party’s name. Finally click call/answer button, the phone dial out NOTE 1: When omitting select an idle channel key and filling in the number directly on the main screen, the software will assign a free channel for the call automatically. NOTE 2: When user fill in number and make a call under software client TE mode, system will records this number as answering party’s hotkey automatically by default. If the hotkey number doesn’t exist, system will switch to PSTN mode automatically and regard it as telephone number to dial. |
| Telephone | Web phone or telephone | The client doesn’t enable the phone password: Mode 1: First fill in the voice interface card’s port number of server. After the server is appropriate, follow the voice prompt to enter calling party’s extension phone number, answering party’s TECARE hotkey and end with “#” in turn. The phone dial out. Mode 2: First press “Ringing dial” button the calling party’s telephone will ring. Pick up telephone and follow the voice prompt to dial out the answering party’s TECARE No. by ending with “#”. The dial out. The client enable the phone password: Mode 1: First fill in the voice interface card’s port number of server. After the server is appropriate, follow the voice prompt to enter calling party’s extension phone number, answering party’s TECARE hotkey and calling party’s phone password by ending with “#” in turn,. The phone dial out. Mode 2: First press “ringing dial” button, the calling party’s telephone will ring. Pick up phone and follow the voice prompt to dial out the answering party’s TECARE No. by ending with “#”. The phone dial out. NOTE: If the dialing is incorrect, system will show an error message after dialing completely. User need to restart operation from entering calling party’s extension number step by following the prompts. |

Sheet 1

    - TECARE Answer Party Instruction

When a call comes in, there are two following answer modes.

- Answer by phone. Answer mode: Pick up phone and press “#” to answer.

- Answer by software phone. The mode of reception is divided into three according to the state of incoming calls in different channels

- One channel for incoming call

When one channel calls coming, the TECARE client icon  on the computer screen trap will pop up the caller’s message alert (automatically disappear soon). Double click the icon to open client main interface. The calling channel is shown as yellow. Answering method: click call/ answer button, the phone will be connect in TE mode.

- Multiple channels for incoming calls

When multiple channels calls coming, the TECARE client icon  on the computer screen trap will pop up the last caller’s message alert. Double click the icon to enter client main interface. All calling channels have shown as yellow. Click on each channel, user will get each channel’s calling message.

Answering Method: Click the channel key which is going to be answered firstly. Then click call/ answer button, the phone will be connected in TE mode.

- Missed calls

When there’s a missed call, the TECARE client icon  on the computer screen trip will pop up a red exclamation point. Double click the icon and click call records button to check missed call.

- Consulting and purchasing

Official website: www.teware.net

www.tecare.net

TECARE: Sales@tecare.net

TEL: +86 (0411) 39839800